# BusConnects General Arrangement Drawings

Ballymun/Finglas To City Centre Scheme

# Sheet 01: St. Margaret’s Road / Northwood

Key Features

- Corridor begins at St. Margaret’s Road, transitioning into Northwood. Dedicated bus lanes in each direction.

- New Bus Stop Arrangements on northbound and southbound lanes.

- Continuous segregated cycle tracks in both directions.

- Upgraded signalised junctions at St Margarets Road and Northwood to enhance pedestrian/cyclist safety.

- Raised table side entry treatments at junctions.

- Integration with indicative MetroLink Northwood Station.

# Sheet 02: Balbutcher Lane / Santry Avenue / Ballymun Main Street

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Existing signalised junction at Balbutcher Lane/ Santry Avenue upgraded for pedestrian and cyclist safety.

- New bus stop arrangements.

- Parking removed at multiple frontages.

- Existing toucan crossing retained.

# Sheet 03: Ballymun Car Park / Coultry Road / Shangan Road

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Existing signalised junction at Ballymun Road/ Shangan Road upgraded for pedestrian and cyclist safety.

- Speed limit change 70m north of Shangan Road: - inbound speed limit to 30km/h,  outbound speed limit to 50km/h

- Integration with Indicative MetroLink Ballymun Station.

- New parallel parking provisions created along Ballymun Main Street, with new tree lines provided in both sides, in footpaths

- New bus stops arrangements adjacent to Ballymun Shopping Centre.

- Proposed toucan crossing introduced.

- Use of raised crossings at side roads.

# Sheet 04: Ballymun Civic Centre / Shanliss Road / Trinity Comprehensive

Key Features

- New bus stops arrangements on northbound and southbound lanes.

- Continuous segregated cycle tracks in both directions.

- Speed limit change 150m south of Gateway Crescent - inbound speed limit to 50km/h, outbound speed limit to 30km/h.

- New parallel parking created along Ballymun Main Street.

- Several raised tables for traffic calming and side‑road protection.

- Existing signalised junctions upgraded for pedestrian and cyclist safety.

# Sheet 05: Ballymun Library / Collins Avenue Junction

Key Features

- New bus stops arrangements on northbound and southbound lanes.

- Continuous segregated cycle tracks in both directions.

- Library access narrowed to improve pedestrian/cyclist safety.

- New outbound bus stop arrangement, located south of Glasnevin Avenue Junction.

- Existing signalised junction at Glasnevin Avenue/ Collins Avenue upgraded for pedestrian and cyclist safety.

# Sheet 06: Ballymun Road / St Pappin Road / Albert College

Key Features

- New bus stops arrangements on northbound and southbound lanes, with existing removed.

- Continuous segregated cycle tracks in both directions.

- New parking bays introduced adjacent outbound carriageway.

- Additional toucan crossings

- Existing signalised junction at St Pappin Rd upgraded for pedestrian and cyclist safety.

- Use of raised crossings at Albert College Drive.

- Integration with Indicative MetroLink Collins Avenue Station.

# Sheet 07: DCU / St Canice’s Road / Hampstead Avenue

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Existing parking removed adjacent inbound carriageway.

- New bus stops arrangements to replace existing stops.

- Raised tables on St Canice’s Road and Hampstead Avenue side roads.

- Existing St Canice Road junction upgraded to signalised control to enhance cyclist and pedestrian facilities.

# Sheet 08: Griffith Avenue / St. Mobhi Road

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Griffith Avenue converted in 2-way arrangement.

- Rearranged Griffith Avenue/ Ballymun Road, St Mobhi Road - One lane eastbound directing traffic from Ballymun Rd Inbound and Griffith Ave Eastbound. One turning traffic lane directing traffic from St Mobhi Rd Outbound and Griffith Ave Westbound onto Ballymun Rd Outbound. One westbound traffic lane. Bus lane provided between both (lane 2) to facilitate turning buses onto Ballymun Rd Outbound.

- Ban left turn from St Mobhi Rd Inbound onto Griffith Avenue Eastbound

- Ballymun Road converted in 2-way section. One bus lane outbound and 1 traffic lane per direction.

- Northbound bus gate implemented on St Mobhi Road with time‑based restrictions.

- New bus stop arrangements on Ballymun Rd and St Mobhi Rd.

- Multiple toucan crossings, new and retained.

- Raised table interventions across The Rise and DCU Sportsground side roads.

- Existing signalised junctions upgraded at Griffith Avenue/ Ballymun Rd/ St Mobhi Rd for pedestrian and cyclist safety

# Sheet 09: Home Farm Road / Stella Avenue

Key Features

- Dedicated bus lanes southbound direction only.

- Continuous segregated cycle tracks in both directions.

- Northbound bus gate implemented on St Mobhi Road with time‑based restrictions.

- New bus stops at both carriageway edges, with some existing bus stops removed.

- New Raised table crossings at side roads and existing toucan crossing retained at Scoil Chaitriona.

# Sheet 10: St Mobhi Road / St Mobhi Drive / Botanic Avenue

Key Features

- Dedicated bus lanes southbound direction only.

- Continuous segregated cycle tracks in both directions. Two-way track adjacent inbound carriageway from Scoil Chaitriona to St Mobhi Drive.

- Existing rail alongside Na Fianna to be relocated. Existing wall alongside Home Farm FC to be demolished, with new retaining wall required.

- New two-way cycle track along south side of St Mobhi Drive, adjacent to the Tolka River.

- One new and one existing toucan crossing retained.

- Northbound bus gate implemented on St Mobhi Road with time‑based restrictions.

- Junction upgrade at Botanic Avenue to improve pedestrian/cyclist protection.

- Integration with Indicative MetroLink Griffith Park Station.

# Sheet 11: Botanic Road / Fairfield Road / Marguerite Road

Key Features

- Dedicated bus lanes southbound direction only up to Botanic Road Junction.

- No bus lane provision in either direction between St Mobhi Road/ Botanic Road Junction to south of Botanic Road / Margurette Road Junction.

- Continuous segregated cycle tracks in both directions

- Three new bus stops plus removals of older stops.

- Raised table treatments at numerous side entry points.

- Advance signage for bus gate – general traffic diverted northbound via Botanic Road.

- Junction upgrade at Botanic Road/ St Mobhi Road to improve pedestrian/cyclist protection.

# Sheet 12: Botanic Road / Prospect Way / Finglas Road

Key Features

- Dedicated bus lanes in each direction from south of Botanic Road / Margurette Road Junction.

- Continuous segregated cycle tracks in both directions.

- Two-way cycle track proposed along north side of Prospect Way.

- Two-way cycle track proposed along east side of Botanic Road inbound from Prospect Way junction

- New bus stop arrangements

- Pedestrian crossings removed where corridor realignment occurs.

- Turning lane onto Finglas Road relocated,

- providing new landscaped pocket. Straight movement from Lindsay Rd to Finglas Road not permitted.

- Multiple no-turn restrictions to maintain one-way flows on Finglas Road and Botanic Road.

# Sheet 13: Prospect Road / Royal Canal Bank / Phibsborough Road

Key Features

- Dedicated bus lanes in each direction

- Existing Prospect Road bridge widened over railway line to accommodate new 2-way cycle track.

- New bridge crossing over railway line adjacent to existing Westmoreland Bridge to accommodate new 2-way cycle track.

- New bridge crossing over Royal Canal to accommodate new 2-way cycle track.

- Primary cycle route follows Quiet Street routing along Royal Canal Bank,

- New bus stop arrangements near future Metrolink Glasnevin station.

- Raised table treatments at side roads along Phibsborough Road.

# Sheet 14: Phibsborough Road / North Circular Road

Key Features

- Dedicated bus lanes in each direction along Phibsborough Road.

- Underpass for cycle route (following quiet street routing) and pedestrians at interface with North Circular Road, providing continuity  from Phibsborough Library to southern section of Royal Canal bank. Landscaped area in the underpass and depressed space.

- Public realm improvements around Irish Volunteers Statue.

- New bus stop arrangements along Phibsborough Road.

# Sheet 15: Phibsborough Road / Royal Canal Bank / Broadstone Depot

Key Features

- Dedicated bus lanes in each direction along Phibsborough Road

- Primary cycle route follows Quiet Street routing along Royal Canal Bank

- New bus stop arrangements along Phibsborough Road.

- Raised table traffic calming at side roads.

# Sheet 16: Broadstone Gate/ Western Way / Constitution Hill

Key Features

- Dedicated bus lanes in each direction

- Primary cycle route follows Quiet Street routing, before joining Western Way and linking back to Constitution Hill.

- Continuous segregated cycle tracks in both directions from Broadstone Gate along Constitution Hill.

- Two-way cycle track on east side of Constitution Hill form Broadstone Gate to Coleraine Street

- LUAS line interface near Broadstone Gate.

- New bus stop arrangements outside Dublin Bus Depot.

- Toucan crossings, raised tables, and signal control for bus priority.

- Landscaped pocket garden at Broadstone Bus depot. And landscaping amenity area adjacent the LUAS line at Western Way.

# Sheet 17: New Street North / St. Michan’s / King Street North

Key Features

- Dedicated bus lanes in each direction – inbound bus lane stops at Church street Upper/ King St North junction

- Continuous segregated cycle tracks in both directions (outbound cycle lane starts at Church street Upper/ King St North junction)

- Two-way cycle track on east side of Constitution Hill form Broadstone Gate to Coleraine Street – access to cycle route on Quiet Street routing along Coleraine Street, Linenhall Street and Ann Street North

- New bus stops added; existing stops removed.

- Proposed signalised junction at Linenhall Street/ King Streer North and upgrades to existing Church Street Upper junction with Brunswick Street North and King Street North.

- Raised table treatments at side roads and some traffic calming features on quiet streets.

# Sheet18: Church Street / Arran Quay

Key Features

- Dedicated bus lanes in each direction – inbound from Church Avenue West to the Quays.

- Short sections where bus lanes discontinues – bus avail of signal control priority to join general traffic lanes.

- Short sections of cycle lanes in each direction – cyclist to use bus lanes where provided.

- New bus stop arrangements along Church Street.

- LUAS line interface at Chancery Street.

- Existing Garda parking retained at Bridewell Station.

- Raised table treatments at side roads.

# Sheet 19: George’s Hill / St Michan’s Street / Ormond Square/ Charles Street

Key Features

- Quiet street treatment continues to provide central city cycling route.

- LUAS line interface at Chancery Street.

- Existing street furniture/ bollards to be adapted to allow cycle traffic through.

# Sheet 20: Ballymun Road / Glasnevin Hill

Key Features

- Rearrangement of road marking at Old Ballymun Road, between Griffith Avenue and Church Avenue, to provide street parking line on the East side and Outbound cycle lane on the west side, where the width of the road is sufficient.

- Section of Old Ballymun Road, between Claremont Avenue and Church Avenue, banned for Outbound traffic, providing footpath extension at the southern end of this section to narrow the road to one lane.

- Outbound cycle track in all section, inbound cycle track between St David’s Terrace and Constitution Hill.

# Sheet 21: Glasnevin Hill

Key Features

- Rearrangement of road at Glasnevin Hill between Old Ballymun Rd to St Mobhi Drive, with minor alterations to the kerb alignment to facilitate new inbound cycle track.

- Existing street parking lines retained, provision of new street parking spaces in front of Bon Scours Hospital.

- Bon Secours Hospital access rearranged to reduce width, providing additional pedestrian space.

- Traffic calming measures proposed to increase safety. New pedestrian crossing at Bon Secours Hospital Access.

# Sheet 22: Glasnevin Hill / St Mobhi Drive / Botanic Road

Key Features

- Junction with St Mobhi Drive rearranged to provide Cycle facilities

- New fully segregated 2-way cycle track along green area between St Mobhi drive and Tolka river.

- Modify St Mobhi drive western end to facilitate cycle track approach to St Mobhi Drive and adapt to Westbound traffic ban at the junction.

- Public ream improvement at Botanic avenue.

- Footpath extension to reduce junction width, and improvement of Public Space.

# Sheet 23: St. Margaret’s Road / Casement Road / Barry Avenue

Key Features

- Corridor begins at Finglas Rd / St. Margaret’s Road Roundabout. Dedicated bus lanes in each direction along Finglas Road.

- New bus stop arrangements on both sides of Finglas Road.

- Proposed pedestrian crossings provided at all arms approaching Finglas Rd / St Margarets Rd roundabout.

- Footpath connection introduced between Casement Road and new bus stop on Finglas Rd (outbound).

- Open existing wall for pedestrian access from North Road to new bus stop on Finglas Rd (inbound).

# Sheet 24: Finglas North / Casement Road / Brookville

Key Features

- Dedicated bus lanes in each direction.

# Sheet 25: Finglas North / Mellowes Park

Key Features

- Dedicated bus lanes in each direction.

# Sheet 26: Finglas Village / Mellowes Road

Key Features

- Dedicated bus lanes in each direction.

- New cycle track on outbound carriageway terminating at Mellows Rd.

- Road widening on Mellowes road junction exit ramp, to facilitate new bus lane and cycle track from Church Street to Mellowes road

- Road widening on Mellowes road junction entry ramp, to facilitate new bus lane (inbound).

# Sheet 27: Finglas / Wellmount Road / Church Street

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- New bus stop arrangements on Finglas Road.

- Existing signalised junctions upgraded for pedestrian/cyclist facilities at Church St and Wellmount Road.

- Existing pedestrian bridge retained at Church Street.

- Open wall/rail improvements for pedestrians from Church Street to access new bus stop on Finglas Road (inbound).

- Proposed toucan crossing added.

# Sheet 28: Finglas Road/ Finglas Place

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Junction upgrade at Finglas Road and Finglas Road/ Finglas Place junctions to improve pedestrian/cyclist protection.

- New  bus stops arrangements along Finglas Road.

# Sheet 29: Finglas Road / Clearwater Shopping Centre / Glenhill Road

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- New bus stop arrangements at Clearwater Shopping Centre.

- Existing Finglas Road/ Glenhill Road signalised junction upgraded to enhance pedestrian/cyclist safety.

# Sheet 30: Finglas Road / Premier Square / The Griffith / Ardmore Hotel

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- New bus stops on both sides of Finglas Road adjacent the Ardmore Hotel (outbound) and Premier Square (inbound).

- Existing Finglas Road/ The Griffith signalised junction upgraded to enhance pedestrian/cyclist safety.

- Raised table side‑entry treatments for side roads.

# Sheet 31: Prospect Hill / Tolka Valley Road / Craigie Court

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Existing Finglas Road/ Tolka Valley Road signalised junction upgraded to enhance pedestrian/cyclist safety.

# Sheet 32: Finglas Road / Old Finglas Road

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Tie‑in with Tolka Valley Cycle Route.

- Existing Finglas Road/ Old Finglas road signalised junction upgraded to enhance pedestrian/cyclist safety.

- New toucan crossing connecting Finglas Road to Toka Valley cycle route.

- New bus stop arrangements both directions.

# Sheet 33: Finglas Road / Ballyboggan Road / Glasnevin Cemetery

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Existing Finglas Road/ Ballyboggan Road signalised junction upgraded to enhance pedestrian/cyclist safety.

- Existing slip lanes removed at Ballyboggan Road junction.

- New bus stops, alongside removal of some existing stops.

# Sheet 34: Glasnevin Cemetery / Slaney Road / LIDL / Petrol Station

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

# Sheet 35: The Willows / Church of Jesus Christ of Latter-Day Saint

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- New bus stops on both carriageway edges.

- Existing Finglas Road/ Slaney Road signalised junction upgraded to enhance pedestrian/cyclist safety.

- Existing Finglas Road junction with The Willows upgraded to signalised junction to enhance pedestrian/cyclist safety.

- Raised table side entry treatments for side roads.

# Sheet 36: Claremont Lawns / Claremont Court / Glasnevin Cemetery

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Existing Finglas Road/ Claremount Court signalised junction upgraded to enhance pedestrian/cyclist safety.

- Proposed toucan crossing added adjacent to Claremount Lawns.

- New bus stop arrangements at Glasnevin Cemetery.

- Raised table entries at side roads.

- Road widening into public park with existing parking locations remodelled. New green verges and footpath.

# Sheet 37: Prospect Way / Prospect Avenue / Finglas Road

Key Features

- Dedicated bus lanes in each direction.

- Continuous segregated cycle tracks in both directions.

- Road widening into footpath to facilitate new bus lane and cycle track requiring land take at St Vincent School.

- Existing Finglas Road/ Prospect Way signalised junction upgraded to enhance pedestrian/cyclist safety.

- New bus stop arrangements in both directions

- Raised table side‑entry treatments at side roads.

- Proposed toucan crossing at St Vincents Secondary School.